DEPARTMENT OF COMPUTER SCIENCE & INFORMATION TECHNOLOGY

BACHELORS OF SCIENCE IN COMPUTER SCIENCE

Knowledge Based System (CT-359)

Knowledge-Based Expert System for Centrifugal Pump
Fault Diagnosis and Maintenance Recommendation

Project Report

Group Members: ______________________________
Instructor: _________________________________
Submission Date: 18 October 2026

# Abstract

This project develops an explainable Knowledge-Based System (KBS) for diagnosing faults in a centrifugal pump and recommending maintenance actions. The prototype represents heterogeneous equipment knowledge through weighted production rules linking components, symptoms, faults, likely causes, safety priorities, repair cost, downtime, and maintenance actions. Numeric operating observations are transformed into symbolic evidence and processed using forward-chaining style rule evaluation plus weighted evidence ranking. The system returns multiple candidate faults with confidence-style evidence scores, an explanation trace, and prioritized maintenance recommendations. The project includes 14 fault classes, 22 symptoms, 16 maintenance actions, 14 compound rules, 280 synthetic diagnostic cases, and a 255-record synthetic sensor log. On the internally generated diagnostic cases, the prototype achieved 85.7% top-1 and 98.2% top-3 agreement with the generating label; these figures demonstrate internal consistency only and are not a claim of real-world diagnostic accuracy.

# 1. Problem Definition and Objectives

Industrial pump knowledge is typically distributed across maintenance manuals, operator observations, historical records, expert experience, and sensor readings. The proposed KBS brings these sources into a structured knowledge model so that maintenance personnel can enter observed evidence and receive a transparent diagnostic recommendation.

Represent equipment, components, symptoms, faults, causes, evidence, and maintenance actions in a machine-readable form.

Analyze stakeholder needs, incomplete requirements, operational constraints, and safety priorities.

Compare alternative knowledge-representation and inference approaches and justify the selected method.

Implement a working prototype capable of handling incomplete or overlapping evidence.

Provide explainable ranked diagnoses and maintenance recommendations.

# 2. CLO and Complex-Problem Mapping

| Outcome / Attribute | How addressed |
| --- | --- |
| CLO 1 – Knowledge Representation | Components, symptoms, faults, maintenance actions, weighted relationships, and production rules are stored in CSV/JSON knowledge bases. |
| CLO 2 – Problem Analysis & Solution Design | Stakeholders, functional/non-functional requirements, incomplete information, safety, downtime, cost, and production continuity are analyzed and reflected in the architecture. |
| CLO 3 – Method & Algorithm Assessment | Rule-based, ontology/knowledge-graph, case-based, and data-driven approaches are compared. Weighted forward reasoning is selected for transparency and manageable data requirements. |
| CPA-2 | There is no single obvious diagnosis when multiple faults share symptoms; the system ranks competing hypotheses. |
| CPA-3 | The solution combines KBS concepts, pump domain knowledge, Python implementation, structured data, and reasoning. |
| CPA-9 | Unclear sensor thresholds, missing observations, inconsistent evidence, and stakeholder trade-offs are explicitly identified. |

# 3. Selected Equipment: Centrifugal Pump

A centrifugal pump was selected because it has well-defined hydraulic, mechanical, electrical, and sealing subsystems, and many faults exhibit overlapping but explainable symptom patterns. This makes it suitable for a knowledge-based diagnostic prototype.

| Component | Category | Role |
| --- | --- | --- |
| Pump Casing | Hydraulic | Contains fluid and converts velocity to pressure. |
| Impeller | Hydraulic | Transfers rotational energy to the fluid. |
| Shaft | Mechanical | Transmits torque from driver to impeller. |
| Bearings | Mechanical | Support shaft and control radial/axial movement. |
| Mechanical Seal | Sealing | Limits leakage along the rotating shaft. |
| Coupling | Mechanical | Connects motor shaft to pump shaft. |
| Suction Line / Strainer | Hydraulic | Supplies liquid to pump inlet. |
| Discharge Line / Valve | Hydraulic | Controls pump discharge flow. |
| Electric Motor | Electrical | Provides mechanical drive power. |
| Baseplate / Foundation | Structural | Maintains alignment and structural rigidity. |

# 4. Stakeholders and Requirements

## 4.1 Stakeholders

| Stakeholder | Primary need |
| --- | --- |
| Maintenance technician | Likely fault, cause, inspection steps, repair priority, explanation. |
| Operator | Simple symptom entry and immediate warning when a dangerous pattern appears. |
| Maintenance engineer | Traceable rules, editable knowledge base, alternative diagnoses. |
| Production manager | Reduced downtime and visibility of maintenance impact. |
| Safety personnel | Conservative warnings and safe isolation recommendation for critical conditions. |
| Management | Lower unplanned maintenance cost and consistent decision support. |

## 4.2 Functional Requirements

Accept numeric sensor observations and manual symptoms.

Convert numeric readings into symbolic evidence.

Match evidence against stored fault knowledge.

Rank multiple possible faults instead of forcing a single answer.

Show confidence/evidence score, safety priority, cost, and downtime.

Recommend maintenance actions.

Show which evidence and rules caused each conclusion.

## 4.3 Non-Functional Requirements

Explainability and traceability.

Fast interactive response.

Knowledge base should be editable without rewriting the entire program.

Clear separation between knowledge base and inference engine.

Educational safety warning: prototype thresholds are not manufacturer limits.

## 4.4 Incomplete / Ambiguous Requirements

The assignment does not specify pump model, duty point, fluid properties, motor rating, sensor type, or OEM alarm limits.

Real diagnostic thresholds vary by machine and service; therefore the prototype uses clearly labeled academic demo thresholds.

Conflicting evidence can occur because faults share symptoms such as vibration, low flow, and heating.

Not every symptom is always observable; therefore the inference engine uses partial evidence rather than requiring a complete rule match.

# 5. Knowledge Representation Design

The project uses a hybrid rule-based representation. Atomic relationships are stored as weighted fault–symptom links. Higher-order patterns are represented as production rules whose conditions must all be observed before the rule fires. Metadata for safety, repair cost, downtime, and maintenance actions is stored separately so recommendations remain modular.

| Entity | Examples | Storage |
| --- | --- | --- |
| Equipment / components | Impeller, bearing, seal, coupling, suction line, motor | components.csv |
| Symptoms / evidence | High vibration, low flow, seal leakage, crackling noise | symptoms.csv |
| Faults | Cavitation, bearing failure, misalignment, dry running | faults.csv |
| Weighted relationships | Fault F02 ↔ suction_pressure_low weight 5 | symptom_fault_weights.csv |
| Compound rules | IF crackling_noise AND suction_pressure_low AND pressure_fluctuation THEN cavitation | rules.json |
| Maintenance actions | Inspect bearing, restore prime, clean strainer | maintenance_actions.csv |

# 6. Fault Knowledge Base

| ID | Fault | Priority | Cost | Downtime |
| --- | --- | --- | --- | --- |
| F01 | Bearing Wear / Failure | HIGH | Medium | Medium |
| F02 | Cavitation | HIGH | Low | Medium |
| F03 | Shaft Misalignment | HIGH | Medium | Medium |
| F04 | Impeller Blockage / Fouling | MEDIUM | Low | Medium |
| F05 | Mechanical Seal Failure | HIGH | Medium | Medium |
| F06 | Suction Blockage / Clogged Strainer | HIGH | Low | Low |
| F07 | Dry Running / Loss of Prime | CRITICAL | Medium | High |
| F08 | Motor Overload | HIGH | Medium | Medium |
| F09 | Coupling Damage / Looseness | HIGH | Medium | Medium |
| F10 | Impeller Damage / Erosion | HIGH | High | High |
| F11 | Discharge Restriction / Closed Valve | HIGH | Low | Low |
| F12 | Air Ingress on Suction Side | HIGH | Low | Medium |
| F13 | Base / Foundation Looseness | MEDIUM | Medium | Medium |
| F14 | Electrical Supply Imbalance | HIGH | Medium | Medium |

# 7. Example Production Rules

R01: IF vibration_high AND bearing_temp_high AND grinding_noise
     THEN Bearing Wear / Failure

R02: IF crackling_noise AND suction_pressure_low AND pressure_fluctuation
     THEN Cavitation

R07: IF no_flow AND prime_failure AND rapid_temp_rise
     THEN Dry Running / Loss of Prime

R12: IF air_bubbles AND flow_fluctuation AND pressure_fluctuation
     THEN Air Ingress on Suction Side

# 8. Inference Mechanism

The inference process is forward and evidence-driven. The user provides facts. Numeric values are first converted into symbolic evidence. Each candidate fault receives points for matched evidence. If every condition of a compound rule is satisfied, its bonus is added. Candidate faults are then ranked by normalized score, followed by safety priority as a tie-breaker.

For a fault f, the prototype computes:

Score(f) = Σ matched symptom weights + Σ fired rule bonuses

Evidence Score (%) = 100 × Score(f) / Maximum configured score(f)

This percentage is an explainable evidence score for the prototype, not a calibrated probability. The design is intentional: it allows incomplete evidence and competing diagnoses without hiding the reasoning in a black-box model.

# 9. Why This Method Was Selected

| Method | Strength | Limitation | Decision |
| --- | --- | --- | --- |
| Rule-based / forward reasoning | Highly explainable; works with expert knowledge; little training data required | Rule maintenance can grow with system size | Selected |
| Backward chaining | Good when testing one known hypothesis | Less natural for broad symptom-first screening | Discussed, not primary |
| Ontology / knowledge graph | Excellent semantic relationships and extensibility | More tooling and modeling overhead for this prototype | Future extension |
| Case-based reasoning | Useful with historical maintenance cases | Requires a meaningful library of validated cases | Future extension |
| Machine learning | Can model complex patterns from large datasets | Needs reliable labeled data; explanation may be weaker | Not selected for core inference |

# 10. System Architecture

Operator / Sensor Inputs
         |
         v
Numeric-to-Symbolic Evidence Layer
         |
         v
+---------------- Knowledge Base ----------------+
| Components | Symptoms | Faults | Weights | Rules|
| Safety | Cost | Downtime | Maintenance Actions  |
+--------------------------------------------------+
         |
         v
Inference Engine
(weighted matching + forward rule firing)
         |
         v
Ranked Faults -> Explanation Trace -> Maintenance Recommendation

# 11. Dataset and Logging Design

| File | Size / contents | Purpose |
| --- | --- | --- |
| diagnostic_cases.csv | 280 labeled synthetic cases × 22 symptom fields | Tests incomplete/overlapping symbolic evidence. |
| operational_sensor_log.csv | 255 five-minute synthetic records | Demonstrates time-stamped vibration, temperature, pressure, flow, current and labeled fault episodes. |
| evaluation_results.csv | 280 predictions | Stores top prediction and top-3 inclusion result for each diagnostic case. |
| components.csv | 10 components | Equipment structure. |
| faults.csv | 14 faults | Fault metadata and priorities. |
| symptoms.csv | 22 symptoms | Controlled vocabulary for evidence. |
| maintenance_actions.csv | 16 actions | Repair/inspection guidance. |
| rules.json | 14 compound rules | Explicit explainable IF–THEN patterns. |

Important: the diagnostic cases and sensor logs are synthetic and reproducible. They were created for academic prototyping, software testing, and demonstration. They are not real maintenance records and must not be presented as field measurements.

# 12. Prototype Thresholds

The software contains demo thresholds to convert numeric observations into symbolic evidence, for example vibration ≥ 6.0 mm/s RMS → vibration_high, bearing temperature ≥ 80°C → bearing_temp_high, and flow < 75% of a healthy baseline → flow_low. These values are implementation defaults for the academic demonstration only. A real deployment would load model-specific operating envelopes and alarm limits from the manufacturer or plant engineering database.

# 13. Evaluation

The inference engine was evaluated on the 280 internally generated symbolic diagnostic cases. The result was 85.7% top-1 agreement and 98.2% top-3 inclusion of the generating fault label. Because the cases were generated from the same knowledge model, this evaluation checks implementation consistency rather than independent field validity. Real accuracy must be validated on expert-reviewed maintenance cases from the target plant.

| Metric | Result | Interpretation |
| --- | --- | --- |
| Top-1 agreement | 85.7% | The highest-ranked diagnosis matched the synthetic generating label in 240/280 cases. |
| Top-3 inclusion | 98.2% | The generating label appeared among the first three candidates in 275/280 cases. |
| Automated unit tests | 5/5 passed | Representative bearing, cavitation, dry-running, air-ingress, and sensor-conversion cases passed. |

# 14. Demonstration Scenarios

| Scenario | Evidence | Expected top diagnosis |
| --- | --- | --- |
| Bearing distress | High vibration + hot bearing + grinding/rumbling noise | Bearing Wear / Failure |
| Cavitation | Crackling noise + low suction pressure + pressure fluctuation + low flow | Cavitation |
| Dry running | Near-zero flow + loss of prime + rapid temperature rise | Dry Running / Loss of Prime |
| Air ingress | Air bubbles + unstable pressure/flow | Air Ingress on Suction Side |
| Discharge restriction | High discharge pressure + low flow + rapid heating | Discharge Restriction / Closed Valve |

# 15. Safety, Cost, Downtime and Explainability

Each fault is associated with safety priority, qualitative repair cost, and qualitative downtime. The interface highlights critical conditions, while the explanation trace shows matched evidence and rule identifiers. This supports accountable decision-making: the user can see not only what the system recommends, but why the recommendation appeared.

# 16. Limitations and Future Work

Knowledge is manually engineered and limited to the selected centrifugal-pump fault set.

The evidence score is not a statistical probability and is not calibrated on field data.

Synthetic logs do not capture all real process disturbances, simultaneous faults, sensor bias, or transient behavior.

A real deployment should integrate OEM-specific thresholds, historian/SCADA data, maintenance work orders, and expert validation.

Future versions can add an ontology/knowledge graph, temporal rules, case-based reasoning, probabilistic confidence, and live sensor integration.

# 17. Conclusion

The project demonstrates how a KBS can transform fragmented industrial maintenance knowledge into a structured, explainable fault-diagnosis tool. The weighted production-rule approach is appropriate for the assignment because it supports knowledge representation, problem analysis, method comparison, incomplete evidence, alternative diagnoses, and transparent maintenance recommendations while remaining understandable enough to defend during a viva.

# References

[1] KSB, “Maintenance – Centrifugal Pump Lexicon.” https://www.ksb.com/en-global/centrifugal-pump-lexicon/article/maintenance-1116120

[2] KSB, “Dry running – Centrifugal Pump Lexicon.” https://www.ksb.com/en-global/centrifugal-pump-lexicon/article/dry-running-1116984

[3] KSB, “Operating behaviour – Centrifugal Pump Lexicon.” https://www.ksb.com/en-global/centrifugal-pump-lexicon/article/operating-behaviour-1118222

[4] KSB, “Pipeline pump – Centrifugal Pump Lexicon.” https://www.ksb.com/en-global/centrifugal-pump-lexicon/article/pipeline-pump-1116452

[5] Project source files and synthetic datasets supplied with this submission.